IoT Lab Report

LoRa-Based Environmental Monitoring System

Using LoRa (Long Range Radio) Communication

IIT Patna – IoT Research Laboratory

# 1. Objective

Design, implement, and validate a wireless sensor network for real-time environmental monitoring using LoRa at 433.5 MHz. A transmitter node equipped with a BME680 and MQ-135 sensor encodes multi-parameter data into a 24-byte binary packet and transmits it to a receiver node, which decodes readings and logs link quality metrics (RSSI and SNR).

# 2. Hardware Used

| Component | Role | Qty |
| --- | --- | --- |
| Heltec WiFi LoRa 32 (V3) | Transmitter & Receiver node (ESP32 + SX1262) | 2 |
| BME680 Sensor | Temperature, Humidity, Pressure, Gas Resistance | 1 |
| MQ-135 Gas Sensor | CO2 / Air Quality Index estimation | 1 |
| USB Cable | Power and programming | 2 |

# 3. Circuit Diagram & Pin Configuration

Figure 1: Wiring connections for BME680 and MQ-135 sensors to Heltec V3

| Heltec V3 Pin | Sensor Pin | Description |
| --- | --- | --- |
| GPIO 40 (SDA) | BME680 SDA | Data wire – I2C |
| GPIO 41 (SCL) | BME680 SCL | Clock wire – I2C |
| 3V3 | BME680 VCC | High voltage (power) |
| GND | BME680 GND | Ground |
| GPIO 7 (ADC) | MQ-135 AOUT | Analog data wire |
| 5V | MQ-135 VCC | High voltage (power) |
| GND | MQ-135 GND | Ground |

# 4. LoRa Configuration

The SX1262 radio chip is configured for maximum range using SF12 over 433.5 MHz ISM band with hardware CRC. A 24-byte packed binary struct carries all sensor fields with no ASCII overhead.

| Parameter | Value | Remarks |
| --- | --- | --- |
| RF Frequency | 433.5 MHz | ISM sub-GHz band (Asia) |
| Spreading Factor | SF12 | Maximum range, longer air time |
| Bandwidth | 125 kHz | Better sensitivity, lower data rate |
| Coding Rate | 4/8 | Maximum forward error correction |
| TX Output Power | 14 dBm (~25 mW) | Sufficient for lab/outdoor range |

# 5. Sensor Working Principles

## BME680

Four-in-one Bosch sensor communicating via I2C (address 0x77). Measures temperature (±1 °C), humidity (±3 %), pressure (0.18 Pa resolution), and VOC gas resistance. Gas heater runs at 320 °C / 150 ms. Oversampling: 8×T, 4×P, 2×H with IIR filter of size 3.

## MQ-135 CO2 Estimation

Semiconductor sensor whose SnO2 layer changes resistance with target gas concentration. CO2 ppm is estimated via the power-law:

PPM = 110.47 × (Rs / Ro)^(−2.86)

Rs = sensor resistance derived from 12-bit ADC reading; Ro = 2.7 kΩ calibrated in clean air. Note: MQ-135 is cross-sensitive to multiple gases — values represent relative air quality, not absolute CO2.

# 6. Observations & Results

| Parameter | Typical Value | Sensor |
| --- | --- | --- |
| Temperature | 25–35 °C | BME680 |
| Relative Humidity | 40–70 % | BME680 |
| Pressure | 1000–1013 hPa | BME680 |
| CO2 (estimated) | 400–1000 ppm | MQ-135 |
| RSSI (lab range) | -30 to -60 dBm | SX1262 report |
| SNR | > 10 dB | SX1262 report |

In the lab at close range (< 10 m), RSSI was typically −30 to −60 dBm with SNR > 10 dB — well above the SF12 sensitivity floor of −137 dBm. Binary struct memcpy() ensured bit-perfect float reconstruction. CRC at hardware level discarded any corrupted packets.

# 7. Conclusion & Future Enhancements

The experiment demonstrated a complete IoT pipeline: sensor acquisition → 24-byte binary LoRa transmission → receiver decoding with RSSI/SNR logging. Key takeaways:

- SF12 + BW 125 kHz provides ~+20 dB link budget vs SF7 at the cost of ~2 s air time per packet.

- Packed binary encoding is 3–5× more efficient than ASCII CSV for LoRa payloads.

- BME680 gas resistance and MQ-135 CO2 together give a complementary VOC + air quality picture.

Planned enhancements include: cloud upload via HTTP POST (code stub already present), LoRaWAN OTAA integration, deep-sleep duty cycling, AES-128 payload encryption, GPS tagging, and migration to a dedicated NDIR CO2 sensor (SCD41) for absolute accuracy.

# 8. References

- Semtech SX1262 Datasheet – Long Range LoRa Modem, Rev 2.1

- Bosch BME680 Datasheet – Environmental Sensor, Rev 1.6

- Winsen MQ-135 Gas Sensor Datasheet

- Heltec WiFi LoRa 32 V3 Schematic – heltec.org

- LoRa Alliance LoRaWAN Specification v1.0.4